PRODUCT & SYSTEM ENGINEERING REPORT

Universal Two-Wheeler Rider Safety System

PS1 — Intelligent Rider Safety Prototype

Creating intelligent devices to improve the commutation sector

| Document item | Details |
| --- | --- |
| Project | Universal Two-Wheeler Rider Safety System |
| Prototype platform | ESP32 + MPU6050 + NEO-6M GPS + smartphone application |
| Mobile platform | iOS / SwiftUI |
| Device communication | Bluetooth Low Energy (BLE) |
| Primary purpose | Accident, riding-behaviour, overspeed and personal-safety response |
| Design principle | Small, affordable, retrofit-ready and useful for every rider |

# 1. Executive Summary

The Universal Two-Wheeler Rider Safety System is an integrated safety platform intended for two-wheeler riders of all ages, genders and riding purposes. The concept combines accident detection, speed monitoring, unstable-riding detection, emergency SOS, controlled guardian location sharing and safe-arrival notification into one retrofit-ready system.

The system is divided into two cooperating parts: a compact bike-mounted safety unit and a smartphone application. The safety unit collects sensor and GPS information and communicates with the phone over BLE. The smartphone provides the user interface, location services, emergency workflow and communication layer.

The first prototype focuses on demonstrating the complete device-to-phone pathway and the core safety logic without requiring a custom PCB, GSM module or full cloud backend.

# 2. Problem Statement

Two-wheelers are highly exposed to road and personal-safety risks. The original concept report identifies four major situations: accidents that may go unnoticed, unsafe or unstable riding, overspeeding and personal safety threats. Existing products generally address only one of these areas.

The proposed system therefore treats these situations as different safety scenarios handled by one common platform rather than creating separate products for separate groups of riders.

# 3. Product Philosophy

The system is designed for every two-wheeler rider.

It is not limited to a women's-safety use case.

It is not only a GPS tracker.

It is not only an accident detector.

It combines multiple safety scenarios and responds according to the rider's situation.

Guardian tracking is explicitly controlled by the rider rather than being permanently active.

This preserves the central product identity: a Universal Two-Wheeler Rider Safety System that observes the riding situation and provides an appropriate response.

# 4. Target Users and Use Cases

| User group | Relevant safety need |
| --- | --- |
| Students | Accident response, guardian check-in, safe arrival |
| Daily commuters | Overspeed and unstable-riding warnings, emergency assistance |
| Delivery riders | Continuous ride monitoring and incident response |
| Elderly riders | Accident detection and rapid emergency notification |
| Any rider | Physical/app SOS and controlled location sharing |

# 5. System Behaviour

| Detected situation | System response |
| --- | --- |
| Normal riding | Monitor quietly in the background |
| Overspeeding | Warn the rider and retain the relevant safety record |
| Prolonged unstable riding | Warn the rider and display a 'Take Rest' safety message |
| Hard impact / accident condition | Ask 'Are you OK?' and start a 30-second confirmation window |
| No response within 30 seconds | Escalate to the emergency workflow |
| Rider feels threatened | Physical or in-app SOS |
| Ride reaches destination | Safe-arrival notification |
| Guardian check-in | Opt-in live location sharing |

# 6. Overall System Architecture

TWO-WHEELER RIDER
                           |
                           v
              +-------------------------+
              | Bike-mounted Safety Unit|
              |        ESP32            |
              +-----------+-------------+
                          |
          +---------------+----------------+
          |               |                |
          v               v                v
      MPU6050           NEO-6M          SOS Input
   Motion/Impact          GPS            Button
          |               |                |
          +---------------+----------------+
                          |
                          v
                    BLE Communication
                          |
                          v
              +-------------------------+
              |      iPhone App         |
              |       Ride Safe         |
              +-----------+-------------+
                          |
       +------------------+-------------------+
       |                  |                   |
       v                  v                   v
   Ride/Safety       Location & Speed    Emergency UI
    Monitoring          Processing       / Contacts
       |                  |                   |
       +------------------+-------------------+
                          |
                          v
                Emergency / Guardian
                    Communication

The prototype architecture deliberately keeps the device simple. The ESP32 acts as the embedded controller and BLE communication endpoint, while the iPhone handles the higher-level user interface and phone-based communication.

# 7. Hardware Architecture

| Component | Purpose |
| --- | --- |
| ESP32 DevKit | Main microcontroller, sensor processing and BLE communication |
| MPU6050 | Accelerometer and gyroscope data for impact and motion analysis |
| NEO-6M GPS | Location and GPS-based speed information |
| Push button | Physical SOS input |
| Buzzer | Local rider warning / emergency feedback |
| LED + 220Ω resistor | Visual status indication |
| Breadboard + jumper wires | Prototype interconnection |
| USB / power source | Prototype power and programming |

## 7.1 Prototype Wiring

| Module / signal | ESP32 pin |
| --- | --- |
| MPU6050 VCC | 3V3 |
| MPU6050 GND | GND |
| MPU6050 SDA | GPIO 21 |
| MPU6050 SCL | GPIO 22 |
| GPS TX | GPIO 16 |
| GPS RX | GPIO 17 |
| SOS button | GPIO 25 → GND |
| LED | GPIO 26 through 220Ω |
| Buzzer | GPIO 27 + GND |

## 7.2 Hardware Validation

During prototype development, the MPU6050 was independently verified on I2C address 0x68 and successfully produced changing accelerometer and gyroscope values. This confirmed that the sensor and I2C wiring were functional. The development process also separates sensor debugging from BLE/app integration so that a temporary sensor communication issue does not prevent the BLE connection from being demonstrated.

# 8. Bluetooth Low Energy Communication

The ESP32 exposes a BLE service used by the Ride Safe iPhone application. The device advertises under the name PS1_Safety_Device.

| BLE item | Value |
| --- | --- |
| Device name | PS1_Safety_Device |
| Service UUID | 6E400001-B5A3-F393-E0A9-E50E24DCCA9E |
| RX UUID | 6E400002-B5A3-F393-E0A9-E50E24DCCA9E |
| TX UUID | 6E400003-B5A3-F393-E0A9-E50E24DCCA9E |
| Communication | ESP32 ↔ iPhone over BLE |

The prototype uses command messages such as:

CANCEL   → cancel an active accident confirmation
SOS      → trigger app/device SOS workflow
RESET    → reset the current safety state

The ESP32 also transmits status information in a structured line format:

STATUS,GPS=0,LAT=0.000000,LON=0.000000,SPEED=0.00,ACCEL=0.00,GYRO=0.00,SOS=0,ACCIDENT=0

# 9. Embedded Software

The ESP32 firmware is implemented with the Arduino framework under PlatformIO. It initializes BLE, GPS, MPU6050, SOS input, LED and buzzer, then continuously processes sensor and safety states.

| Development item | Configuration |
| --- | --- |
| Project folder | PS1_Prototype |
| Framework | Arduino |
| Board | ESP32 Dev Module / esp32dev |
| Platform | Espressif32 |
| Serial monitor | 115200 baud |
| Development environment | PlatformIO + VS Code |
| USB serial port used | /dev/cu.usbserial-10 |

## 9.1 Embedded Safety Logic

Read motion, GPS and input data.

Calculate relevant motion/rotation conditions.

Monitor speed and riding behaviour.

Detect a hard-impact/accident condition when configured thresholds are reached.

Start the rider confirmation window.

If the rider cancels, return to normal monitoring.

If there is no response within 30 seconds, escalate to the emergency workflow.

Allow the physical SOS button to trigger immediate SOS behaviour.

Use the buzzer/LED for local feedback.

Transmit current status to the smartphone over BLE.

# 10. Mobile Application — Ride Safe

The iOS application is built with SwiftUI and is intended to be the main user-facing safety interface. It communicates with the ESP32 over BLE and uses iPhone capabilities for location and phone-based emergency actions.

| Application component | Role |
| --- | --- |
| ContentView.swift | Main application navigation / root view |
| DashboardView.swift | Current system and safety status |
| RideView.swift | Ride monitoring interface |
| EmergencyView.swift | Emergency controls and information |
| HistoryView.swift | Safety/ride history interface |
| SettingsView.swift | User preferences and safety controls |
| BLEManager.swift | ESP32 BLE scanning, connection and data exchange |
| LocationManager.swift | iPhone location handling |
| SafetyManager.swift | Safety-state and incident logic |
| EmergencyManager.swift | Emergency action handling |
| StorageManager.swift | Local persistence |
| Models.swift | Application data models |

## 10.1 App Permissions

Bluetooth permission is required to communicate with the ESP32.

Location permission is required for location-based safety functions.

Phone-based emergency communication must follow iOS system restrictions and user permissions.

## 10.2 Physical iPhone Prototype

The application has been built for and tested on a physical iPhone 15. For the final demonstration, the physical device is preferred because it provides actual Bluetooth and location behaviour rather than simulator-only behaviour.

# 11. Core Safety Workflows

## 11.1 Accident Workflow

Hard impact / abnormal motion detected
                |
                v
        "Are you OK?" prompt
                |
          30-second window
          /              \
       YES                NO / no response
        |                       |
        v                       v
 Cancel / normal          Emergency escalation
 monitoring              through phone workflow

## 11.2 Physical SOS Workflow

Rider presses physical SOS button
                |
                v
        SOS event generated
                |
                v
     Smartphone emergency workflow
                |
       +--------+--------+
       |                 |
   Rider contact      Emergency service
      pathway          pathway

The prototype supports the physical SOS concept through the ESP32 input and app emergency workflow. Actual SMS/calling is subject to iOS permissions and system restrictions; the application does not silently bypass those restrictions.

## 11.3 Unstable Riding Workflow

Motion data
    |
    v
Sustained abnormal pattern
    |
    v
Rider warning
    |
    v
"Take Rest" safety message
    |
    v
Record / notify according to configured safety policy

## 11.4 Overspeed Workflow

GPS speed
    |
    v
Configured speed threshold
    |
    +---- below threshold ----> Normal monitoring
    |
    +---- above threshold ----> Rider warning
                                  |
                                  v
                           Safety record / alert

## 11.5 Guardian Tracking Workflow

Rider enables Guardian Tracking
             |
             v
     Location sharing active
             |
      +------+------+
      |             |
  During ride    Continuous
      |             |
      +------+------+
             |
             v
      Rider disables it
             |
             v
       Sharing stops

# 12. End-to-End Data Flow

MPU6050 ──────┐
               │
GPS ───────────┼──> ESP32 ──BLE──> iPhone Ride Safe App
               │                         │
SOS Button ────┘                         ├── Dashboard
                                         ├── Ride monitoring
                                         ├── Safety decisions
                                         ├── Location services
                                         ├── Emergency UI
                                         ├── History / records
                                         └── Guardian / notification workflow

# 13. Prototype Component Cost

| Component | Prototype cost |
| --- | --- |
| Breadboard | ₹50 |
| LEDs | ₹6 |
| MPU6050 | ₹270 |
| NEO-6M GPS | ₹320 |
| Male-to-Male jumper wires | ₹15 |
| Female-to-Male jumper wires | ₹15 |
| Female-to-Female jumper wires | ₹15 |
| 220Ω resistors | ₹10 |
| ESP32 | ₹380 |
| Push button | ₹3 |
| Buzzer | ₹15 |

Estimated listed component subtotal: ₹1,099.

This is the component list used for the prototype estimate. It does not include the cost of an iPhone, development computer, custom enclosure, custom PCB, manufacturing, or future communication/cloud services.

# 14. Privacy and Safety Considerations

Guardian location is opt-in and rider-controlled.

Location sharing can be restricted to ride periods.

The system should not become a permanent tracking mechanism by default.

Emergency communication is initiated through the rider's phone and must respect platform restrictions.

The prototype should clearly distinguish a detected incident from a confirmed emergency.

Emergency contacts should be configured by the rider before use.

Safety alerts should provide clear, actionable information rather than unnecessary notifications.

# 15. Current Prototype Status

| Area | Current status |
| --- | --- |
| ESP32 firmware | Implemented in PlatformIO / Arduino |
| BLE device | Configured as PS1_Safety_Device |
| BLE service/characteristics | Configured with fixed UUIDs |
| MPU6050 | Hardware/I2C independently verified |
| GPS | Integrated into firmware and status data path |
| SOS button | Integrated into firmware |
| Buzzer + LED | Integrated for local feedback |
| iOS app | SwiftUI project built for physical iPhone 15 |
| App BLE layer | Implemented |
| Dashboard / Ride / Emergency / History / Settings | Implemented in prototype app structure |
| Emergency phone actions | Implemented subject to iOS restrictions |
| Full production backend | Not part of first prototype |
| Custom PCB / GSM | Deferred |

# 16. Demonstration and Testing Procedure

Connect the ESP32 to the Mac by USB.

Open the PlatformIO project PS1_Prototype.

Build and upload the firmware to the ESP32.

Open the serial monitor at 115200 baud.

Confirm that BLE advertising starts and the device name is PS1_Safety_Device.

On the physical iPhone 15, enable Bluetooth permission for Ride Safe.

Open Ride Safe and start the device scan.

Select PS1_Safety_Device when it appears.

Confirm the ESP32 serial monitor reports an iPhone BLE connection.

Observe the app receiving status information.

Demonstrate ride/safety status, SOS and emergency UI using controlled test conditions.

For accident testing, use the software/demo controls or safe simulated input rather than intentionally creating a dangerous impact.

Important prototype note: the ESP32 can advertise and establish BLE communication independently of the MPU6050. Therefore, BLE/app integration can be demonstrated while a separate sensor issue is being debugged.

# 17. Current Limitations

The prototype is not a certified automotive or medical safety device.

GPS accuracy depends on satellite visibility and the NEO-6M module.

BLE requires the smartphone to be within communication range.

Automatic SMS/calling behaviour is constrained by iOS permissions and system policies.

The first prototype does not use a dedicated GSM module.

No custom PCB or production enclosure is included in the first prototype.

Advanced cloud/backend infrastructure is deferred.

Accident and unstable-riding thresholds require field calibration before real-world deployment.

False positives and false negatives remain possible until the algorithms are validated with representative riding data.

# 18. Future Scope

Custom PCB and compact weather-resistant enclosure.

Improved motion/impact classification using calibrated sensor fusion.

More robust unstable-riding detection using longitudinal ride data.

Improved GPS filtering and speed estimation.

Dedicated communication hardware for deployments where phone connectivity is insufficient.

Cloud/guardian dashboard if required and designed with explicit consent.

Advanced geofencing and route-based safety rules.

More detailed ride analytics and incident history.

Power optimization for long-duration operation.

Field trials across different two-wheelers and riding conditions.

Manufacturing and cost optimization for retrofit deployment.

# 19. Requirements-to-Implementation Mapping

| Requirement | Prototype implementation |
| --- | --- |
| Universal rider coverage | Common platform and safety scenarios for all riders |
| Bluetooth connection | ESP32 BLE service + iPhone BLEManager |
| Location tracking | GPS/device data + iPhone location layer |
| GPS speed | NEO-6M data and speed processing |
| Accident detection | MPU6050 motion/impact data + confirmation workflow |
| 30-second cancellation | Accident confirmation timer |
| Multiple contacts | Emergency contact data model/UI |
| Physical SOS | ESP32 button input |
| App SOS | In-app emergency control |
| Rider warnings | Phone UI + buzzer/LED |
| Safe arrival | Application ride-state workflow |
| Geofencing | Application-level safety feature |
| Guardian control | Explicit tracking ON/OFF concept |
| Retrofit-ready | External compact prototype intended for existing two-wheelers |
| Affordable | Low-cost off-the-shelf prototype components |

# 20. Conclusion

The Universal Two-Wheeler Rider Safety System combines embedded sensing, BLE communication, GPS, a physical SOS interface and an iPhone application into a single rider-safety concept. The design is intentionally universal: accident detection, unstable riding, overspeeding, personal safety and guardian check-in are treated as different situations within the same platform.

The prototype architecture provides a practical path from concept to demonstration. The ESP32 handles the bike-side sensing and BLE communication, while the iPhone provides the user interface, location services and phone-based emergency workflow. The resulting system remains aligned with the original constraints of being small, affordable, retrofit-ready and useful for every rider.

# Appendix A — Key Technical Reference

| Parameter | Value |
| --- | --- |
| ESP32 I2C SDA | GPIO 21 |
| ESP32 I2C SCL | GPIO 22 |
| GPS RX | GPIO 16 |
| GPS TX | GPIO 17 |
| SOS | GPIO 25 |
| LED | GPIO 26 |
| Buzzer | GPIO 27 |
| MPU6050 address | 0x68 |
| BLE device | PS1_Safety_Device |
| Serial speed | 115200 |
| Accident confirmation | 30 seconds |

# Appendix B — Project Development Principle

The development sequence should remain modular: validate the hardware independently, validate ESP32 firmware, validate BLE communication, validate the iPhone application, and then combine the safety workflows. This makes fault isolation easier and allows the prototype demonstration to continue even when an individual sensor or feature is temporarily under debugging.